1. Abstract

The 129Knots BDN Validation System is an end-to-end automated pipeline designed to detect maritime fraud and verify regulatory compliance in Bunker Delivery Notes (BDNs). The system ingests raw BDN documents in any format digital PDF or handwritten scans and produces a structured verdict with a 0–100% confidence score within 5–20 seconds.

The platform combines Optical Character Recognition (OCR), vessel and barge identity resolution against external maritime registries, AIS (Automatic Identification System) geolocation verification, MARPOL Annex VI fuel compliance checks, and an ML-based anomaly detector into a single 9-phase pipeline exposed via a FastAPI REST interface and a browser-based investigation dashboard.

2. System Architecture

3. Technology Stack

| Technology | Description |
| --- | --- |
| Technology | Description |
| Python | Primary programming language used for implementing the entire validation pipeline. |
| FastAPI | Built RESTful APIs for document upload, processing, validation, and report generation. |
| Uvicorn | ASGI server used to run and serve the FastAPI application. |
| PostgreSQL | Stored document metadata, extracted information, validation results, and audit logs. |
| SQLAlchemy | ORM used for database modeling and interaction with PostgreSQL. |
| Alembic | Managed database schema migrations and version control. |
| Tesseract OCR | Extracted text from scanned and photographed Bunker Delivery Notes (BDNs). |
| OpenCV | Preprocessed document images using denoising, deskewing, thresholding, and contour detection to improve OCR accuracy. |
| Pillow (PIL) | Performed image loading, resizing, and format conversion during document processing. |
| PyMuPDF | Converted PDF documents into high-resolution images for OCR and extracted embedded PDF content. |
| RapidFuzz | Performed fuzzy string matching to identify field names and correct OCR inconsistencies. |
| spaCy | Extracted structured entities such as vessel names, IMO numbers, bunker suppliers, and locations when OCR output was inconsistent. |
| scikit-learn (Isolation Forest) | Detected anomalous or potentially fraudulent bunker transactions based on extracted document features. |

4. Validation Pipeline

Confidence Score Composition

| Component | What It Measures |
| --- | --- |
| Document Credibility (35%) | OCR text confidence, field completeness ratio, MARPOL fuel spec checks, timestamp integrity, font consistency, correction keyword detection |
| Vessel Identity (25%) | Registry match depth, IMO resolution success, MMSI retrieval, identity conflict flags |
| AIS Anomaly Score (40%) | Vessel–barge separation distance, speed during delivery, AIS gap duration, port proximity, co-location overlap percentage |

Verdict Thresholds

| Verdict | Criteria |
| --- | --- |
| VALID | ≥75% All checks passed. Low fraud risk. |
| SUSPICIOUS | 45–74% Minor flags present. Human review recommended. |
| HIGH RISK | <45% Serious anomalies detected. Review required. |
| APPROVED | Any Manually approved by a reviewer after inspection. |
| REJECTED | Any Manually rejected by a reviewer after inspection. |